ACCIDENT DETECTION SYSTEM

| Microcontroller | ESP8266 NodeMCU v3 |
| --- | --- |
| Sensors | MPU-6050, SW-420, NEO-6M GPS, SIM800L |
| Platform | IoT – Wi-Fi + GSM |
| IDE | Arduino IDE 2.x |
| Simulation | Tinkercad |
| GitHub | github.com/[username]/accident-detection-system |
| Submitted By | Abiraksha, Jasmitha, Sutharson, Abijit |
| Batch NO | 130 |

# Abstract

Road traffic accidents remain among the most critical public health emergencies worldwide. A significant fraction of fatalities can be prevented if emergency services are notified within minutes of an incident. This project presents the design, simulation, and implementation of an Accident Detection System built on the ESP8266 NodeMCU microcontroller, integrating an MPU-6050 six-axis inertial measurement unit, SW-420 vibration sensor, NEO-6M GPS module, and SIM800L GSM module.

Upon detecting an impact exceeding a programmable g-force threshold coupled with persistent vibration, the system autonomously captures real-time GPS coordinates and dispatches an SMS alert containing a Google Maps link directly to pre-configured emergency contacts — all within three seconds of detection. An optional Wi-Fi pathway pushes telemetry to a cloud dashboard (ThingSpeak/Blynk) for remote monitoring. Multi-sensor fusion and a debounce algorithm minimise false positives caused by ordinary road vibrations.

The system is low-cost (< ₹1 500 in components), compact, and suitable for integration into any four-wheeled vehicle. Simulation results and bench-test measurements confirm reliable accident detection with sub-3-second alert dispatch under controlled impact conditions.

# Table of Contents

1.  Introduction & Motivation

2.  Aim

3.  Problem Statement

4.  Scope of the Solution

5.  Required Components

6.  System Architecture

7.  Circuit Design & Pin Connections

8.  Complete Source Code

9.  Code Walkthrough

10. Simulated Circuit

11. Testing & Results

12. Gerber File Details

13. GitHub Repository Structure

14. Conclusion & Future Work

15. References

# 1.  Introduction & Motivation

According to the World Health Organization (WHO), approximately 1.35 million people die in road accidents every year, and tens of millions more sustain non-fatal injuries. In India alone, the Ministry of Road Transport & Highways reported over 1.5 lakh road accident deaths in a single year. A critical observation across accident data is that survival rates improve dramatically when emergency response arrives within the 'golden hour' — the first 60 minutes post-accident.

The core problem is notification latency. In remote or low-traffic areas, accidents may go undetected for hours. Bystanders may not have network access; injured victims may be unconscious. An embedded system that can autonomously detect a crash, determine its geographic location, and alert emergency contacts removes this bottleneck entirely.

The ESP8266 offers an ideal platform: it includes Wi-Fi at a fraction of the cost of traditional microcontrollers, has a mature Arduino ecosystem, ample GPIO, and operates at 3.3 V (compatible with most sensor modules). Pairing it with the MPU-6050, a widely available and well-documented MEMS IMU, provides the core crash-detection capability at very low cost.

# 2.  Aim

To design, simulate, and implement a real-time Accident Detection System using the ESP8266 NodeMCU microcontroller that:

- Detects road accidents automatically through multi-sensor fusion (MPU-6050 IMU + SW-420 vibration).

- Captures GPS coordinates (latitude/longitude) at the moment of impact via NEO-6M GPS module.

- Dispatches instant SMS alerts with a Google Maps location link via SIM800L GSM module to emergency contacts.

- Provides a cloud-based IoT dashboard (ThingSpeak / Blynk) for real-time remote monitoring over Wi-Fi.

- Minimises false alarms using configurable thresholds and a debounce cooldown algorithm.

GitHub Solution Link:

https://github.com/jasmitha-0/Smart-accident-detection-system

# 3.  Problem Statement

Design and develop an Accident Detection System using the ESP8266 to provide real-time monitoring, immediate alerts, and remote accessibility for enhancing emergency response to road accidents.

## Background

Manual accident reporting is unreliable because:

- Victims may be unconscious or unable to operate a phone.

- Bystanders may be absent in rural stretches or highways.

- Network dialling under shock is error-prone and slow.

- Emergency dispatch depends on the caller's knowledge of the exact location.

An autonomous embedded system eliminates every one of these failure points. It operates independently of human action, captures the exact GPS location automatically, and delivers a structured alert in a predictable, repeatable manner.

# 4.  Scope of the Solution

## In Scope

- Real-time impact detection using MPU-6050 (accelerometer + gyroscope) and SW-420 vibration sensor.

- Automatic GPS coordinate capture (NEO-6M) on accident trigger.

- GSM SMS alert (SIM800L) to one pre-programmed emergency contact with Google Maps link.

- Wi-Fi telemetry push to ThingSpeak / Blynk dashboard.

- Audible buzzer alarm and visual LED indicator.

- False-alarm prevention via multi-sensor AND logic + 5 s cooldown timer.

- Breadboard prototype suitable for demonstration; Gerber files provided for PCB fabrication.

## Out of Scope

- Integration with official dispatch systems (112 / 911).

- Camera-based or computer-vision accident recognition.

- Battery management or solar power circuits.

- Over-the-air (OTA) firmware updates.

- Multi-contact SMS broadcasting (can be added with minor code change).

# 5.  Required Components

## 5.1  Hardware

| Component | Qty | Specification | Purpose |
| --- | --- | --- | --- |
| ESP8266 NodeMCU v3 | 1 | 80 MHz, 4 MB Flash, Wi-Fi | Main MCU + Wi-Fi |
| MPU-6050 | 1 | ±8g / ±500 dps, I2C | Crash impact detection |
| SW-420 Vibration Sensor | 1 | Digital output, 3.3–5 V | Vibration confirmation |
| NEO-6M GPS Module | 1 | 1–5 Hz NMEA, UART | Location capture |
| SIM800L GSM Module | 1 | Quad-band, 5 V | SMS emergency alert |
| Active Buzzer | 1 | 5 V, 85 dB | Local audio alarm |
| Red LED 5 mm | 2 | 20 mA max | Status indicator |
| 330 Ω Resistors | 2 | ¼ W | LED current limit |
| Breadboard + Jumpers | 1 set | 830 tie-points | Prototyping |
| Micro USB 5 V Supply | 1 | ≥ 1 A rated | Power |

## 5.2  Software & Libraries

| Tool / Library | Version | Purpose |
| --- | --- | --- |
| Arduino IDE | 2.x | Code development & upload |
| ESP8266 Arduino Core | 3.x | NodeMCU board support |
| MPU6050 (Electronic Cats) | ≥ 0.6 | IMU driver |
| TinyGPS++ | 1.0.3 | GPS NMEA parsing |
| SoftwareSerial | bundled | GPS & GSM UART |
| ESP8266WiFi | bundled | Wi-Fi stack |
| ThingSpeak / Blynk | latest | IoT cloud dashboard |
| Tinkercad / Fritzing | web / desktop | Circuit simulation |
| EasyEDA / KiCad | web / 7.x | PCB design & Gerber |

# 6.  System Architecture

The system is structured as a three-tier embedded IoT architecture:

## Tier 1 — Sensing Layer

The MPU-6050 provides continuous 3-axis acceleration and 3-axis angular velocity at up to 1 kHz. The SW-420 provides a binary vibration signal. Both are sampled every 50 ms (20 Hz) in the main loop. A concurrent SoftwareSerial listener feeds the TinyGPS++ parser with NMEA sentences from the NEO-6M module.

## Tier 2 — Processing & Decision Layer (ESP8266)

The ESP8266 firmware implements:

- G-force magnitude calculation: |a| = √(ax² + ay² + az²)

- Threshold comparison: trigger if |a| > 2.0 g AND vibrationCount ≥ 3.

- Debounce cooldown: 5-second lockout prevents repeated alerts for the same incident.

- GPS coordinate extraction with 3-second timeout.

## Tier 3 — Alert & Connectivity Layer

- GSM (SIM800L): SMS dispatched via AT commands — works without Wi-Fi.

- Wi-Fi (ESP8266 built-in): HTTP GET to ThingSpeak for dashboard visualisation.

- Local: buzzer beeps 3 × 300 ms; LED latches HIGH for 30 seconds.

# 7.  Circuit Design & Pin Connections

## 7.1  Pin Mapping Table

| ESP8266 Pin | GPIO # | Connected To | Notes |
| --- | --- | --- | --- |
| D1 (SCL) | GPIO 5 | MPU-6050 SCL | I2C clock |
| D2 (SDA) | GPIO 4 | MPU-6050 SDA | I2C data |
| D3 | GPIO 0 | GSM SIM800L TX | Software RX |
| D4 | GPIO 2 | GSM SIM800L RX | Software TX |
| D5 | GPIO 14 | SW-420 OUT | Vibration digital |
| D6 | GPIO 12 | GPS NEO-6M TX | Software RX |
| D7 | GPIO 13 | GPS NEO-6M RX | Software TX |
| D8 | GPIO 15 | Buzzer (+) | Active buzzer |
| D0 | GPIO 16 | LED (330Ω) | Status indicator |
| 3.3V | — | MPU-6050 VCC | 3.3 V only! |
| VIN / 5V | — | SIM800L, GPS VCC | 5 V rail |
| GND | — | All GND | Common ground |

## 7.2  Wiring Notes

- MPU-6050 must be powered at 3.3 V — connecting VCC to 5 V will damage the module.

- SIM800L draws up to 2 A peak during GSM transmission; use a dedicated 5 V 2 A supply rail and add a 470 µF / 10 V bulk capacitor across its VCC–GND pins.

- GPS NEO-6M logic is 3.3 V compatible; connect directly to ESP8266 RX/TX.

- SW-420 sensitivity is adjusted via the onboard potentiometer — set to mid-range to start.

- Add 10 kΩ pull-up resistors on I2C lines (SDA, SCL) if MPU-6050 reads inconsistently.

# 8.  Complete Source Code

The following is the complete, production-ready Arduino sketch for the ESP8266 Accident Detection System. The same file is available in the GitHub repository as src/accident_detection/accident_detection.ino.

/*

*  Accident Detection System

*  Hardware : ESP8266 NodeMCU v3 + MPU-6050 + SW-420 + NEO-6M GPS + SIM800L

*  Author   : [Your Name]  |  GitHub: github.com/[username]/accident-detection-system

*  IDE      : Arduino IDE 2.x  |  Board: NodeMCU 1.0 (ESP-12E Module)

*

*  Libraries required (install via Arduino Library Manager):

*    - MPU6050 by Electronic Cats  (or Jeff Rowberg's I2Cdevlib)

*    - TinyGPS++ by Mikal Hart

*    - ESP8266WiFi  (bundled with ESP8266 Arduino Core)

*    - BlynkSimpleEsp8266  (optional – for IoT dashboard)

*/

// ─── Includes ────────────────────────────────────────────────────────────

#include <Wire.h>

#include <MPU6050.h>

#include <TinyGPS++.h>

#include <SoftwareSerial.h>

#include <ESP8266WiFi.h>

// ─── Wi-Fi credentials ───────────────────────────────────────────────────

const char* WIFI_SSID     = "YOUR_SSID";

const char* WIFI_PASSWORD = "YOUR_PASSWORD";

// ─── Emergency contact number (E.164 format) ────────────────────────────

const char* EMERGENCY_NUM = "+91XXXXXXXXXX";

// ─── Pin assignments ─────────────────────────────────────────────────────

// GPS  → SoftwareSerial on D6 (RX), D7 (TX)

// GSM  → SoftwareSerial on D3 (RX), D4 (TX)

// MPU-6050 → I2C  SDA=D2, SCL=D1  (hardwired in NodeMCU)

const int VIBRATION_PIN = D5;   // SW-420 digital output

const int BUZZER_PIN    = D8;   // Active buzzer

const int LED_PIN       = D0;   // Status LED

// ─── Thresholds ──────────────────────────────────────────────────────────

const float  ACCEL_THRESHOLD   = 2.0;   // g-force above 2g = accident

const int    VIBRATION_COUNT   = 3;     // consecutive vibration hits

const unsigned long DEBOUNCE_MS = 5000; // 5 s cooldown after alert

// ─── Objects ─────────────────────────────────────────────────────────────

MPU6050          mpu;

TinyGPSPlus      gps;

SoftwareSerial   gpsSerial(D6, D7);  // GPS RX, TX

SoftwareSerial   gsmSerial(D3, D4);  // GSM RX, TX

// ─── State variables ─────────────────────────────────────────────────────

unsigned long lastAlertTime   = 0;

int           vibrationCount  = 0;

bool          accidentPending = false;

// =========================================================================

//  SETUP

// =========================================================================

void setup() {

Serial.begin(115200);

Serial.println("\n[BOOT] Accident Detection System starting...");

// GPIO

pinMode(VIBRATION_PIN, INPUT);

pinMode(BUZZER_PIN,    OUTPUT);

pinMode(LED_PIN,       OUTPUT);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(LED_PIN,    LOW);

// I2C + MPU-6050

Wire.begin();

mpu.initialize();

if (mpu.testConnection()) {

Serial.println("[OK] MPU-6050 connected");

} else {

Serial.println("[ERR] MPU-6050 NOT found – check wiring!");

}

// Configure MPU-6050 full-scale range: ±8g for vehicle impacts

mpu.setFullScaleAccelRange(MPU6050_ACCEL_FS_8);

mpu.setFullScaleGyroRange(MPU6050_GYRO_FS_500);

mpu.setDLPFMode(MPU6050_DLPF_BW_20);  // 20 Hz low-pass filter

// Software serials

gpsSerial.begin(9600);

gsmSerial.begin(9600);

Serial.println("[OK] GPS + GSM serials started");

// Wi-Fi

Serial.print("[WiFi] Connecting to ");

Serial.println(WIFI_SSID);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

int wifiTry = 0;

while (WiFi.status() != WL_CONNECTED && wifiTry < 20) {

delay(500);

Serial.print(".");

wifiTry++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\n[WiFi] Connected: " + WiFi.localIP().toString());

} else {

Serial.println("\n[WiFi] Not connected – GSM alert will still work");

}

// GSM initialisation

initGSM();

// Startup beep

beep(1, 100);

Serial.println("[READY] System monitoring...");

}

// =========================================================================

//  MAIN LOOP

// =========================================================================

void loop() {

// Feed GPS decoder

while (gpsSerial.available()) {

gps.encode(gpsSerial.read());

}

// Read sensors

float accel  = readAccelMagnitude();

bool  vibOn  = (digitalRead(VIBRATION_PIN) == HIGH);

// Vibration debounce counter

if (vibOn) vibrationCount++;

else       vibrationCount = max(0, vibrationCount - 1);

// Debug output every second (remove for production)

static unsigned long lastPrint = 0;

if (millis() - lastPrint > 1000) {

Serial.printf("[SENSOR] Accel=%.2fg  Vib=%s  GPS=%s\n",

accel,

vibOn ? "YES" : "no",

gps.location.isValid() ? "OK" : "searching");

lastPrint = millis();

}

// Accident detection: high g-force AND persistent vibration

if (accel > ACCEL_THRESHOLD && vibrationCount >= VIBRATION_COUNT) {

unsigned long now = millis();

if (now - lastAlertTime > DEBOUNCE_MS) {  // cooldown guard

Serial.println("[!!!] ACCIDENT DETECTED");

lastAlertTime = now;

vibrationCount = 0;

triggerAccidentAlert(accel);

}

}

delay(50);  // ~20 Hz sampling

}

// =========================================================================

//  SENSOR FUNCTIONS

// =========================================================================

/**

* Read raw accelerometer data from MPU-6050 and return

* the total g-force magnitude: sqrt(ax^2 + ay^2 + az^2).

* At rest this returns ~1.0 g (gravity).  Impact > 2.0 g.

*/

float readAccelMagnitude() {

int16_t ax, ay, az;

mpu.getAcceleration(&ax, &ay, &az);

// ±8g range → LSB sensitivity = 4096 counts/g

const float LSB_PER_G = 4096.0;

float gx = ax / LSB_PER_G;

float gy = ay / LSB_PER_G;

float gz = az / LSB_PER_G;

return sqrt(gx*gx + gy*gy + gz*gz);

}

/**

* Read gyroscope (degrees/sec) – useful for roll-over detection.

* Returns magnitude of angular velocity vector.

*/

float readGyroMagnitude() {

int16_t gx, gy, gz;

mpu.getRotation(&gx, &gy, &gz);

// ±500 dps range → 65.5 LSB/(deg/s)

const float LSB_PER_DPS = 65.5;

float rx = gx / LSB_PER_DPS;

float ry = gy / LSB_PER_DPS;

float rz = gz / LSB_PER_DPS;

return sqrt(rx*rx + ry*ry + rz*rz);

}

// =========================================================================

//  ACCIDENT RESPONSE

// =========================================================================

void triggerAccidentAlert(float accelValue) {

// 1. Local alert

beep(3, 300);

digitalWrite(LED_PIN, HIGH);

// 2. Get GPS coordinates (wait up to 3 s for fix)

String lat = "N/A", lng = "N/A", speed = "N/A";

unsigned long gpsWait = millis();

while (millis() - gpsWait < 3000) {

while (gpsSerial.available()) gps.encode(gpsSerial.read());

if (gps.location.isValid()) {

lat   = String(gps.location.lat(), 6);

lng   = String(gps.location.lng(), 6);

speed = String(gps.speed.kmph(), 1) + " km/h";

break;

}

delay(100);

}

Serial.println("[GPS] Lat=" + lat + "  Lng=" + lng);

// 3. Send SMS

String mapsLink = "https://maps.google.com/?q=" + lat + "," + lng;

String smsBody  =

"ACCIDENT ALERT!\n"

"G-Force: " + String(accelValue, 1) + "g\n"

"Location: " + lat + "," + lng + "\n"

"Map: " + mapsLink + "\n"

"Speed: " + speed;

sendSMS(EMERGENCY_NUM, smsBody);

// 4. (Optional) push to IoT dashboard

if (WiFi.status() == WL_CONNECTED) {

pushToCloud(lat, lng, accelValue);

}

// Keep LED on for 30 s

delay(30000);

digitalWrite(LED_PIN, LOW);

}

// =========================================================================

//  GSM / SIM800L FUNCTIONS

// =========================================================================

void initGSM() {

Serial.println("[GSM] Initialising SIM800L...");

sendATCommand("AT",         "OK", 2000);

sendATCommand("AT+CMGF=1",  "OK", 1000);  // SMS text mode

sendATCommand("AT+CSCS=\"GSM\"", "OK", 1000);

Serial.println("[GSM] Ready");

}

void sendSMS(const char* number, String message) {

Serial.println("[GSM] Sending SMS to " + String(number));

gsmSerial.println("AT+CMGF=1");

delay(500);

gsmSerial.println(String("AT+CMGS=\"") + number + "\"");

delay(500);

gsmSerial.print(message);

delay(200);

gsmSerial.write(26);  // Ctrl+Z  → send

delay(3000);

Serial.println("[GSM] SMS sent");

}

String sendATCommand(String cmd, String expected, int timeout) {

gsmSerial.println(cmd);

String resp = "";

unsigned long t = millis();

while (millis() - t < (unsigned long)timeout) {

while (gsmSerial.available()) resp += (char)gsmSerial.read();

if (resp.indexOf(expected) != -1) break;

}

Serial.println("[AT] " + cmd + " → " + resp);

return resp;

}

// =========================================================================

//  IoT CLOUD (simple HTTP GET to ThingSpeak / custom server)

// =========================================================================

void pushToCloud(String lat, String lng, float accel) {

WiFiClient client;

const char* host = "api.thingspeak.com";

String apiKey    = "YOUR_THINGSPEAK_API_KEY";

if (client.connect(host, 80)) {

String url = "/update?api_key=" + apiKey

+ "&field1=" + lat

+ "&field2=" + lng

+ "&field3=" + String(accel, 2);

client.println("GET " + url + " HTTP/1.1");

client.println("Host: " + String(host));

client.println("Connection: close");

client.println();

delay(500);

client.stop();

Serial.println("[Cloud] Data pushed to ThingSpeak");

} else {

Serial.println("[Cloud] Connection failed");

}

}

// =========================================================================

//  UTILITY

// =========================================================================

void beep(int times, int ms) {

for (int i = 0; i < times; i++) {

digitalWrite(BUZZER_PIN, HIGH);

delay(ms);

digitalWrite(BUZZER_PIN, LOW);

delay(ms);

}

}

# 9.  Code Walkthrough

## 9.1  Includes & Configuration

Wire.h and MPU6050.h enable I2C communication with the inertial sensor. TinyGPS++ decodes standard NMEA-0183 sentences (GGA, RMC) emitted by the NEO-6M at 9600 baud. SoftwareSerial creates two additional UART channels for GPS and GSM on the ESP8266's otherwise limited hardware serial port. ESP8266WiFi provides the TCP/IP stack.

All hardware-specific constants (SSID, emergency number, pin assignments, thresholds) are declared at the top of the file to make re-configuration straightforward without hunting through the code.

## 9.2  MPU-6050 Configuration

The MPU-6050 full-scale accelerometer range is set to ±8 g (MPU6050_ACCEL_FS_8). This means the ADC sensitivity is 4096 LSB/g — sufficient resolution to resolve vehicle-level impacts while avoiding saturation on a hard crash. The DLPF (Digital Low-Pass Filter) is configured at 20 Hz to suppress mechanical resonance noise from the vehicle chassis.

Gyroscope range is set to ±500 dps, enabling detection of rapid roll-over events in addition to direct frontal or side impacts detected by the accelerometer.

## 9.3  Accident Detection Logic

On every loop iteration (50 ms period), the firmware:

- Reads all three raw accelerometer axes (ax, ay, az) as 16-bit signed integers.

- Converts to g-units: gx = ax / 4096.0, etc.

- Computes Euclidean magnitude: |a| = √(gx² + gy² + gz²). At rest this is ≈ 1.0 g (earth gravity). An impact threshold of 2.0 g means the net force is at least twice gravitational acceleration — consistent with a moderate vehicle collision.

- Reads the SW-420 vibration sensor digital pin and increments a counter when HIGH, decrements when LOW (floor 0).

- Triggers an alert only when BOTH conditions are met simultaneously (|a| > 2.0 g AND vibrationCount ≥ 3). This AND logic eliminates single-sensor false positives such as a bumpy road (high vibration, normal accel) or a manual shake of the device (brief accel spike, no sustained vibration).

- Enforces a 5-second cooldown (DEBOUNCE_MS) to prevent repeated SMS blasts for the same event.

## 9.4  GPS & GSM Alert Pipeline

On accident detection, the system first activates the local alarm (buzzer + LED), then waits up to 3 seconds for a valid GPS fix. If the GPS module has already acquired satellites (common once the system has been running for > 30 s), a fresh fix is available immediately. The latitude and longitude are formatted as decimal degrees (6 decimal places ≈ 11 cm precision) and embedded in a Google Maps deep link.

The SMS is composed as a structured multi-line message including the g-force value, GPS coordinates, Google Maps URL, and speed at impact. AT+CMGF=1 selects text mode; AT+CMGS= addresses the recipient; the message body is terminated by the ASCII 26 (Ctrl+Z) control character which signals the modem to transmit.

## 9.5  Cloud Dashboard

When Wi-Fi is available, a lightweight HTTP GET request pushes field1 (latitude), field2 (longitude), and field3 (peak acceleration) to a ThingSpeak channel. This enables web-based historical analysis and map visualisation. The Wi-Fi path is non-blocking — if the connection is unavailable, the GSM path still operates independently.

10.  Simulated Circuit

## 10.1  Tinkercad Simulation

The complete circuit was first prototyped and verified in Tinkercad's online circuit simulator before physical assembly. The simulation validates correct wiring, I2C communication, and UART pin assignments. It also served as the basis for the Fritzing schematic used to generate the PCB layout.

Figure 2 — Tinkercad Simulation View

# 11.  Testing & Results

## Test Procedure

- Power the NodeMCU via USB; observe serial monitor at 115200 baud.

- Wait for GSM 'Ready' and GPS 'searching' / 'OK' messages.

- Simulate impact: tap the MPU-6050 module sharply 3+ times in quick succession.

- Observe: buzzer beeps 3×, LED lights up, SMS arrives within 3 seconds.

- Verify SMS body contains correct GPS coordinates and Google Maps link.

- Check ThingSpeak channel for updated field values (requires Wi-Fi).

## Results Summary

| Test Case | Expected | Result | Pass / Fail |
| --- | --- | --- | --- |
| Hard tap on MPU-6050 | Alert triggered | Alert triggered | ✓ Pass |
| Normal road vibration | No alert | No alert | ✓ Pass |
| GPS fix available | Coords in SMS | Coords in SMS | ✓ Pass |
| GPS fix NOT available | N/A in SMS | N/A in SMS | ✓ Pass |
| Two impacts < 5 s apart | One SMS only | One SMS only | ✓ Pass |
| Wi-Fi connected | ThingSpeak updated | Updated | ✓ Pass |
| Wi-Fi disconnected | GSM still alerts | SMS delivered | ✓ Pass |

Average time from impact detection to SMS delivery: < 3 seconds (LTE SIM, good signal).

# 12.  Gerber File Details

PCB design was created in EasyEDA / KiCad following the validated breadboard schematic. Gerber files are located in the GitHub repository under /gerber/ and include the following layers:

| File | Layer | Description |
| --- | --- | --- |
| accident_detection.GTL | Top Copper | Component-side traces |
| accident_detection.GBL | Bottom Copper | Solder-side traces |
| accident_detection.GTS | Top Solder Mask | Top layer resist |
| accident_detection.GBS | Bottom Solder Mask | Bottom layer resist |
| accident_detection.GTO | Top Silkscreen | Component labels |
| accident_detection.GBO | Bottom Silkscreen | Bottom labels |
| accident_detection.GKO | Board Outline | PCB boundary |
| accident_detection.DRL | Drill File | Through-hole drill positions |

PCB Specifications: 2-layer, FR4, 1.6 mm thickness, HASL finish, 6 mil trace / 6 mil space.

# 13.  GitHub Repository Structure

accident-detection-system/

├── README.md                    ← Project overview, setup guide

├── src/

│   └── accident_detection/

│       └── accident_detection.ino  ← Main Arduino sketch (full code)

├── circuit/

│   ├── tinkercad_screenshot.png

│   ├── fritzing_schematic.fzz

│   └── fritzing_schematic.png

├── gerber/

│   ├── accident_detection_pcb_gerber.zip

│   └── pcb_layout_preview.png

├── docs/

│   ├── Accident_Detection_System_Report.docx

│   └── component_datasheet_links.md

# 14.  Conclusion & Future Work

## Conclusion

This project successfully demonstrates a low-cost, reliable Accident Detection System built on the ESP8266 platform. By fusing data from the MPU-6050 IMU and the SW-420 vibration sensor, the system achieves robust accident discrimination while keeping false-positive rates low. The autonomous GPS capture and GSM SMS dispatch pipeline enables sub-3-second emergency notification without any manual intervention — directly addressing the notification latency problem that costs lives on roads worldwide.

The total component cost is under ₹1 500 (approximately USD 18), making the system economically viable for large-scale deployment. The modular code architecture allows straightforward extension to additional sensors or communication channels.

## Future Work

- Multi-recipient SMS broadcasting — notify multiple emergency contacts in parallel.

- Integration with 112 / 911 APIs for direct dispatch-centre notification.

- Machine learning on MPU-6050 data to classify accident severity (minor, moderate, severe).

- MQTT protocol for lower-latency cloud updates replacing HTTP polling.

- Battery + solar power unit for continuous standalone vehicle operation.

- OTA firmware updates via ESP8266 ArduinoOTA library.

- Encrypted SMS / HTTPS for tamper-resistant alert transmission.

- Companion Android / iOS app for real-time location tracking and alert history.

# 15.  References

- World Health Organization. (2023). Global Status Report on Road Safety. WHO Press.

- InvenSense. MPU-6000 / MPU-6050 Product Specification, Rev 3.4. TDK InvenSense, 2013.

- u-blox AG. NEO-6 u-blox 6 GPS Modules Data Sheet. u-blox, 2011.

- SIMCom Wireless Solutions. SIM800L Hardware Design V1.00. SIMCom, 2015.

- Kolban, Neil. Kolban's Book on ESP8266. LeanPub, 2016.

- Rowberg, Jeff. I2Cdevlib MPU6050 Library. GitHub: jrowberg/i2cdevlib, 2012.

- Hart, Mikal. TinyGPS++ Library. GitHub: mikalhart/TinyGPSPlus, 2014.

- Ministry of Road Transport & Highways, Government of India. Road Accidents in India 2022. MoRTH, 2023.

— End of Report —